Technical Specification

Ref:  MAR INS SPX 001E		Version number: 1.2

Release date: 02-Dec-2013

Targeted sites: All vessels	Targeted departments: TSG / ALL

Distribution list: CGG Ins., PROLANN

# BROADSEIS SRD, Activation cone Firmware

References:

# Document History

| Version | Date | Written/Modified by | Checked by | Approved by | Comments |
| --- | --- | --- | --- | --- | --- |
| 1.1 | 27-Nov-2013 | D. PONCEAU | P. LE ROUX |  | Modification and correction of draft |
| 1.2 | 02-Dec-2013 | D. PONCEAU | P.LE ROUX | E.HAGET | 1st Released Technical Specification |

# Summary

This document defines specifications for a microcontroller firmware for BroadSeis SRD printed circuit boards. Contractor will provide the firmware, the source code and its documentation in English.

# Table of Content

1	Document History	1

2	Summary	1

3	Table of Content	2

4	BACKGROUND	3

4.1	Power supply	4

4.2	Power consumption management	4

4.3	Battery level monitoring	5

4.4	PCB output control and monitoring	6

4.5	Pressure sensing	7

4.6	Microcontroller programming	7

4.7	LED flashing	7

4.8	Communication with RFID memory	8

4.8.1	Communication protocol	8

4.8.2	Memory format	8

5	FIRMWARE DESCRIPTION	9

5.1	Overview	9

5.2	«Initialization» phase	10

5.3	To read and verify configuration bytes	11

5.3.1	To initialize configuration bytes to default values	11

5.3.2	To read and verify calibration bytes	11

5.3.3	To initialize calibration bytes to default values	11

5.3.4	To initialize internal variables and constants	11

5.3.5	To check status and update ERROR_STATUS	11

5.4	“Stand-by” phase	12

5.4.1	To check status and update ERROR_status	13

5.4.2	To power ON pressure sensing and timer	13

5.4.3	To measure pressure and sensor current level	13

5.4.4	To power ON pressure sensing and timer	13

5.4.5	To compare sensor current level to accepted range	13

5.4.6	To briefly power ON LED output	13

5.4.7	To compare pressure level to OPE_DEPTH	13

5.4.8	To increase STANDBY_CNT and compare it to OPE_COUNT	13

5.4.9	RESET STANDBY_CNT	13

5.4.10	To put microcontroller in ultra-low-power mode	13

5.5	“Operation” phase	14

5.5.1	To read and verify configuration bytes	15

5.5.2	To power ON pressure sensing and timer	15

5.5.3	To measure pressure and sensor current level	15

5.5.4	To power OFF pressure sensing and timer	15

5.5.5	To compare sensor current level to accepted range	15

5.5.6	To compare pressure level to ACT_DEPTH	15

5.5.7	To compare pressure level to OPE_DEPTH	15

5.5.8	To activate PCB output	15

5.5.9	To put microcontroller in ultra-low-power mode	15

5.6	“Error” phase	16

5.6.1	To power ON pressure sensing and timer	17

5.6.2	To measure pressure and sensor current level	17

5.6.3	To power OFF pressure sensing and timer	17

5.6.4	To check ERR_CNT	17

5.6.5	TO RESET ERR_CNT	17

5.6.6	To increase ERR_CNT	17

5.6.7	To compare sensor current level to accepted range	17

5.6.8	Status and update ERROR_status	17

5.6.9	To briefly power ON LED output	17

5.6.10	To put microcontroller in ultra-low-power mode	17

6	BROADSEIS SRD, PCB drawing	18

# BACKGROUND

This document defines specifications for a microcontroller firmware for BroadSeis SRD printed circuit boards. Contractor will provide the firmware, the source code and its documentation in English.

Constants shall be defined and well commented on a specific header file and must be made easily modifiable by a third party.

A synoptique explaining the source code shall be provided at the beginning.

Documentation would include a general architecture schematics and code comments in English

Programming shall be done with an easy distinction between different phases and all comments, including variables shall be in English.

Microcontroller is a PIC16LF1827. It’s an 18 pin device assembled on a Printed Circuit Board. Its behavior is controlled by parameters stored in a memory that can be accessed by RFID (IS0 15693). The microcontroller also stores information on this memory.

## Power supply

Power is supplied to microcontroller via 2 pins.

- Pin 5, VSS: GND network

This pin is connected to PCB ground. It is directly connected to battery pack ground.

- Pin 14, VDD: VDD network

This pin is connected to VDD network. It is connected to main battery through a fuse.

## Power consumption management

Microcontroller use of low-power modes is optimized to reduce power consumption. Microcontroller operations take about 100 ms. in parallel to these operations, it charges an oscillator. As soon as operations are finished, microcontroller is put into low-power mode and oscillator takes about one second to discharge (0.9 sec.). Oscillator’s output is connected to an interrupt input of the microcontroller that wakes it up to begin next phase of operation.

- Pin 9, RB3: LP1-VDD network

This pin charges the oscillator

- PIN 3, RA4 and PIN 6, RB0: Cycle network

These pins are connected to the output of the oscillator that cycles microcontroller operation.

RB0 is configured as an interrupt to wake up the microcontroller from sleep mode when it reaches a low-level (0.15 x VDD).

RA4 is configured as an analog input that can measure oscillator output voltage but is not used anymore in software.

## Battery level monitoring

Battery level monitoring involves three pins:

- Pin 8: RB2/LP0-VDD

This pin is connected to PCB’s LP0-VDD. At high level, it activates the voltage dividers to the battery and the supercapacitor.

- Pin 18: RA1/Vbat1

This pin is connected to the voltage divider to measure battery voltage at the input of the PCB. It is exclusively used as an analog input.

- Pin 2: RA3/Vbat2

This pin is connected to the voltage divider to measure supercapacitor voltage at the input of the PCB. It is exclusively used as an analog input.

Battery level is expected to be within the range [0, 3.7] V resulting in [0, 1.21] V at divider’s output.

When powering on the device, battery level (VDD) will be considered as too low for operation when lower than 3.55 V that’s to say 1.16 V.

ADC voltage reference will be chosen to be 2.048 V resulting in a LSB of 2 mV. As a consequence, normal battery measurement range will be [580, 604] LSB. A lower value would indicate a low battery. A higher value would indicate a measurement error.

Voltage difference between supercapacitor positive pin and battery positive pin is expected to be lower than 20 mV in normal operation. As a consequence, normal supercapacitor measurement range will be [576, 604] LSB and normal level difference will be [0, 4] LSB. A higher value would indicate an abnormal supercapacitor current consumption.

Values stored in RFID memory bytes 32 and 33 are normalized to the maximum value they can reach: 3700 mV or 604 LSB.

| Measurement | Stored value |
| --- | --- |
| 0 to 349 LSB | 0 LSB |
| X from 349 to 604 LSB | X-349 LSB |
| 605 to 1023 LSB | 255 LSB |

If measured value is lower than 349 LSB, stored value is 0x00. If measured value is higher than 604 LSB, stored value is 0xFF.

## PCB output control and monitoring

- Pin 11, RB5: Check-Fire network

This pin activates PCB’s output voltage measurement.

- Pin 16, RA7: FIRE network

This pin is connected to PCB’s FIRE output. It is exclusively used as an output whose level must remain low? except when the activation is wanted.

- Pin 1, RA2: VFIRE network

This pin is connected to the voltage divider to measure voltage at the FB- output of the PCB. It is exclusively used as an analog input.

When “Check-fire” network is activated, output load is placed between FB+ and PCB’s ground in serial with a reference resistor (470 Ω). Measuring the voltage between both ends of this reference resistor allows checking the resistance of the output load that is expected to be within the range [1.5, 3] Ω.

As a consequence, voltage between both ends of this reference resistor should be within the fraction range [99.3, 99.7] % of FB+. FB+ is also a fraction of VDD. Using VDD as a reference for the ADC, it is possible to measure FB+ then voltage between both ends of the reference resistor. The voltage difference between these two resistors has to be within the range [3, 7] LSB.

Last eight bits of measured values are stored in RFID memory bytes 46 and 47.

If measured value is lower than 769 LSB, stored value is 0x00.

## Pressure sensing

Pressure measurement involves 3 pins.

- Pin 9, RB3: LP1-VDD network

This pin activates depth measurement. When sets to high-level, it activates current regulator (U2) that controls the current flowing into the pressure transducer (0.285 mA).

- Pin 12, RB6: ISens-Check network

This pin is connected to a resistor in serial with the pressure transducer to measure the current that it draws. It is used here as an analog input.

As the nominal value of the resistor is 470 Ω and as the nominal current flowing through the pressure transducer is 0.285 mA, the nominal measurement voltage is 0.134 V. As a consequence, this measurement must always be within the range [0.12, 0.15] V; that’s to say within [60, 75] LSB.

- Pin 17, RA0: Ana_Pressure network

This pin is connected to pressure transducer’s amplifier output to measure ambient pressure. It is exclusively used as an analog input.

Current and output measurements are stored periodically in memory: during initialization phase, bytes 36 to 39, and during each PCB check, every 60 cycles in stand-by and operation phases, bytes 40 to 43.

When PCB output is activated, last output measurement is stored in bytes 44 and 45.

## Microcontroller programming

Microcontroller programming uses the technique called “In-Circuit Serial Programming” (ICSP).

It involves 5 pins (including power (VDD) and ground (VSS)) but also:

- Pin 13, RB7: ICSPDAT network.

This connected to ICSPDAT network. It is only used to load firmware in microcontroller.

- Pin 12, RB6: ICSPCLK network.

This connected to ICSPCLK network. It is only used to load firmware in microcontroller.

- PIN 4, RA5

This connected to MCLR/ input of the PCB. It is only used as MCLR/ input (Master Clear with internal pull-up).

It is enabled by clearing LVP bit in second configuration word (“configuration word 2”) or by setting MCLRE bit in first configuration word (“Configuration word 1”).

## LED flashing

Flashing LED is controlled via one pin of the device (Pin 15, RA6). This pin is connected to PCB’s LED output through a resistance used to adjust the current flowing through the LED. It is exclusively used as an output whose level must remain low, except when the LED is wanted to flash.

## Communication with RFID memory

The PCB includes an RFID memory which communicates with the microcontroller through its inter-integrated circuit (I²C) interface. It involves 3 pins:

- Pin 7, RB1: SDA network

This pin is connected to memory’s SDA input. It is used to communicate with the memory.

- Pin 10, RB4: SCLK network

This pin is connected to memory’s SCLK input. It is used to communicate with the memory.

### Communication protocol

On this I²C communication bus, microcontroller is the master, RFID memory is the only slave.

RFID memory is a M24-LR-64. Communication protocol is defined in user manual attached to this document.

### Memory format

Memory format is defined by the following table.

# FIRMWARE DESCRIPTION

## Overview

When the microcontroller is powered on, during a first phase called «INITIALIZATION», the firmware do several routine initializations and checks. If this phase is successful, it goes to «STAND-BY» phase. During «STAND-BY» phase, microcontroller is waiting for a defined sequence of event to go to «OPERATION» phase. During «OPERATION» phase, microcontroller is waiting for a defined sequence of events to activate the output of the PCB or to go back to «STAND-BY» phase. If this activation occurs, it goes to «error» phase.

If an error occurs during all these phases, microcontroller goes to «ERROR» phase during which it stays until errors are cleared. Then it goes back to «INITIALIZATION» phase. During «ERROR» phase, it indicates to the operator this error status.

Microcontroller only stops when powered off.

«ERROR» LED is flashing every cycle 1 sec (0.9 sec) instead of every 3 cycles 3 sec (2.7 sec)

Power is managed by the insertion and removal of the magnetic safety pin.

Magnetic pin «IN»  Power OFF

Magnetic pin «OUT»  power ON

Initialization

Stand-by

Operation

Error

Power ON reset

OK

OK

OK

OK

NOK

NOK

NOK

A watchdog timer checks that each cycle of these phases is performed within the expected duration. In case of any error, an interrupt sends the programm to error phase and a watchdog error bit is updated in RFID memory.

|  | Initialization | Stand-by | Operation | Error |
| --- | --- | --- | --- | --- |
| Watchdog error | 1.3 s | 20 ms. | 20 ms. | 20 ms. |

Periodic checks are done during all these phases to prevent any unexpected activation.

|  | Initialization | Stand-by | Operation | Error |
| --- | --- | --- | --- | --- |
| Output load presence | YES | YES | - | - |
| Pressure measurement | YES | YES | YES | YES |
| Sensor current | YES | YES | YES | YES |
| Timer (1 sec) | YES | - | - | - |
| Main battery | YES | - | - | - |
| Supercapacitor | YES | - | - | - |
| EEPROM | YES | YES | YES | YES |

## «Initialization» phase

When the microcontroller is powered on, during a first phase called «Initialization», the firmware do several routine initializations and checks. If this phase is successful, it goes to «stand by» phase. Otherwise, it goes to «error» phase.

ERROR phase

To initialize internal variables

STAND-BY phase

To read and verify configuration bytes

To check status and update ERROR_STATUS

OK

NOK

To read and verify calibration bytes

To initialize configuration bytes to default values

NOK

OK

To initialize calibration bytes to default values

NOK

OK

To initialize registers and inputs/outputs

Power On reset

ERROR Phase (i.e. EEPROM)

ERROR Phase (i.e. EEPROM)

NOK

NOK

## To read and verify configuration bytes

Microcontroller reads bytes 4 to 7 and 20 to 29 of the memory and compare them with their checksum located in bytes 30 and 31.

### To initialize configuration bytes to default values

Microcontroller accesses memory as soon as available for writing and modifies the fields for serial number, activation depth and filter with their default values.

Serial number: ASCII “0123456789”

Activation depth: 50 m

Filter: 1

### To read and verify calibration bytes

Microcontroller reads bytes 8 to 17 of the memory and compare them with their checksum located in bytes 18 and 19.

### To initialize calibration bytes to default values

Microcontroller accesses memory as soon as available for writing and modifies the fields for calibration parameters with their default values.

CAL_OFFSET: 60 bits

CAL_MEAS0: 239 bits

CAL_MEAS1: 419 bits

CAL_DEPTH0: 50 m

CAL_DEPTH1: 100 m

### To initialize internal variables and constants

Microcontroller computes activation depth equivalent in bits using following formula where X is the activation depth in meters.

ACTDEPTHbits=CALOFFSET+CALMEAS1-CALMEAS0CALDEPTH1-CALDEPTH0 .  ACTDEPTH[m]

OPEDEPTHbits=CALOFFSET+3. CALMEAS1-CALMEAS0CALDEPTH1-CALDEPTH0

Microcontroller also sets values for status limits:

BAT_MIN [580 LSB] and BAT_MAX [604 LSB] are the minimum and maximum acceptable values for measured battery level when powering the device.

FLOT_MIN [3 LSB] and FLOT_MAX [7 LSB] are the minimum and maximum acceptable values for measured difference between FB+ and flotation bag level during normal operation.

SENSOR_MIN [bits] and SENSOR_MAX [bits] are the minimum and maximum acceptable values for measured sensor level during normal operation.

RC_MIN [25600 counts] and RC_MAX [35200 counts] are the minimum and maximum acceptable values for clock level during normal operation.

OPE_DEPTH [bits] and OPE_COUNT [120 count] specifies the transition depth (3 m) and timer (about 2 min (120 cycles) between STAND-BY phase and OPERATION phase

STANDBY_CNT, ACT_CNT and ERR_CNT are local variable integers used for counting cycles. They are initialised at 0.

### To check status and update ERROR_STATUS

Microcontroller measures successively timer duration, battery, flotation bag and sensor levels. It compares these measurements to minimum and maximum acceptable values during normal operation. If any of these measurements is not within the acceptable range, microcontroller goes to “error” phase, otherwise it goes to “stand-by” phase.

At the end of these checks, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field.

Timer duration is measured by charging the oscillator over 10 ms. and counting time needed by RB0 interrupt to trigger the microcontroller. Measured value should be within [0.8, 1.1] s.

Other measurements are described with more details in “ERROR” phase chapter.

## “Stand-by” phase

During «STAND-BY» phase, microcontroller is waiting for a defined sequence of pressure to go to «operation» phase. If an error occurs, microcontroller goes to «error» phase.

ERROR phase

To power OFF pressure sensing and timer

To power ON pressure sensing and timer

To compare sensor current level to acc. range

To wake up on timer interruption

To measure pressure and sensor current level

Increase STANDBY_CNT. Compare it to OPE_CNT (120)

2 min

NOK

Reset STANDBY_CNT

NOK

OK

Over 10 ms.

To check status and update ERROR_STATUS

Every 60 cycles

OPERATION phase

To compare pressure level to OPE_DEPTH (3 m)

If >

To put microcontroller in ultra-low-power mode

If <

If  <

If >

To briefly power ON LED output

OK

OPERATION phase

Every 3 cycles

Reset STANDBY_CNT

INITIALIZATION phase

ERROR phase

Reset STANDBY_CNT

### To check status and update ERROR_status

To minimize power consumption this action is done once every 60 cycles. A local counter will manage this periodicity.

Microcontroller measures successively flotation bag and sensor level. It compares these measurements to minimum and maximum acceptable values during normal operation. If any of these measurements is not within the acceptable range, microcontroller goes to “error” phase, otherwise it goes to “stand-by” phase.

At the end of these checks, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field.

These measurements are described with more details in “ERROR” phase chapter.

Battery level is not checked.

### To power ON pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to positive level.

### To measure pressure and sensor current level

Levels at RA0 and RB6 are measured and average over 100 ms.

### To power ON pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to zero.

### To compare sensor current level to accepted range

Microcontroller compares averaged measurement at RB6 input to accepted range (SENSOR_MIN and SENSOR_MAX). If this measurement is not within the acceptable range, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field. Then, it goes to “error” phase.

### To briefly power ON LED output

This action is done once every 3 cycles. A local counter will manage this periodicity.

Microcontroller turns its FLASH_LED output (RA6) to positive level for 10 ms. and then turns it back to zero again.

### To compare pressure level to OPE_DEPTH

Microcontroller compares averaged measurement at RA0 input to OPE_DEPTH.

### To increase STANDBY_CNT and compare it to OPE_COUNT

If this measurement is greater than OPE_DEPTH, a local counter “STAND-BY_CNT” is increased and compared to OPE_CNT.

### RESET STANDBY_CNT

Microcontroller set STAND-BY_CNT to zero.

### To put microcontroller in ultra-low-power mode

Microcontroller is set to ultra-low-power mode and configured to wake-up when RB0 will reach a low value.

## “Operation” phase

During «OPERATION» phase, microcontroller is waiting for a defined sequence of events to activate the output of the PCB or to go back to «STAND-BY» phase. If an error occurs, microcontroller goes to «ERROR» phase.

ERROR phase

To power OFF pressure sensing and timer

To power ON pressure sensing and timer

To compare sensor current level to acc. range

To wake up on timer interruption

To measure pressure and sensor current level

Increase ACT_CNT and compare to FILTER (default 5)

NOK

Reset ACT_CNT

NOK

OK

Over 10 ms.

To read and verify configuration bytes

ERROR phase

To compare pressure level to ACT_DEPTH (default 50m)

If >

To put microcontroller in ultra-low-power mode

If <

If  <

If >

OK

STAND-BY phase

Reset ACT_CNT

To activate PCB output

To compare pressure level to OPE_DEPTH

If >

If <

ERROR phase

Reset ACT_CNT

### To read and verify configuration bytes

Microcontroller reads bytes 4 to 7 and 20 to 29 of the memory and compare them with their checksum located in bytes 30 and 31.

Microcontroller computes activation depth equivalent in bits using following formula where X is the activation depth in meters.

ACTDEPTHbits=CALOFFSET+CALMEAS1-CALMEAS0CALDEPTH1-CALDEPTH0 .  ACTDEPTH[m]

OPEDEPTHbits=CALOFFSET+3. CALMEAS1-CALMEAS0CALDEPTH1-CALDEPTH0

### To power ON pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to positive level.

### To measure pressure and sensor current level

Levels at RA0 and RB6 are measured and average over 100 ms.

### To power OFF pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to zero.

### To compare sensor current level to accepted range

Microcontroller compares averaged measurement at RB6 input to accepted range (SENSOR_MIN and SENSOR_MAX). If this measurement is not within the acceptable range, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field. Then, it goes to “error” phase.

### To compare pressure level to ACT_DEPTH

Microcontroller compares previously averaged measurement at RA0 input to previously computed ACT_DEPTH.

### To compare pressure level to OPE_DEPTH

Microcontroller compares previously averaged measurement at RA0 input to previously computed OPE_DEPTH.

### To activate PCB output

Microcontroller activates PCB output by setting up RA7 output for 3 seconds.

As a consequence FIRE network is sets up for 3 seconds and PCB output is connected between FB+ and PCB ground.

### To put microcontroller in ultra-low-power mode

Microcontroller is set to ultra-low-power mode and configured to wake-up when RB0 will reach a low value.

## “Error” phase

RESET ERR_CNT

To power OFF pressure sensing and timer

To compare sensor current level to acc. range

To wake up on timer interruption

NO

Over 10 ms.

To power ON pressure sensing and timer

To put microcontroller in ultra-low-power mode

INITIALIZATION phase

STAND-BY phase

OPERATION phase

To increase ERR_CNT

To measure pressure and sensor current level

To compare sensor output level to acc. range

To update STATUS byte in memory

To compare PCB output level to acc. range

ERR_CNT >= 60 ?

STAND-BY phase

STATUS ?

OK

NOK

To briefly power on LED

YES

Watchdog or Software error

### To power ON pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to positive level.

### To measure pressure and sensor current level

Levels at RA0 and RB6 are measured and average over 100 ms.

### To power OFF pressure sensing and timer

Microcontroller turns its LP1-VDD output (RB2) to zero.

### To check ERR_CNT

### TO RESET ERR_CNT

### To increase ERR_CNT

### To compare sensor current level to accepted range

Microcontroller compares averaged measurement at RB6 input to accepted range (SENSOR_MIN and SENSOR_MAX). If this measurement is not within the acceptable range, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field. Then, it goes to “error” phase.

### Status and update ERROR_status

To minimize power consumption this action is done once every 60 cycles. A local counter will manage this periodicity.

Microcontroller measures successively flotation bag and sensor level. It compares these measurements to minimum and maximum acceptable values during normal operation. If any of these measurements is not within the acceptable range, microcontroller goes to “error” phase, otherwise it goes to “stand-by” phase.

At the end of these checks, microcontroller accesses memory as soon as available for writing and updates ERROR_STATUS field.

These measurements are described with more details in “ERROR” phase chapter.

Battery level is not checked.

### To briefly power ON LED output

This action is done once every 3 cycles. A local counter will manage this periodicity.

Microcontroller turns its FLASH_LED output (RA6) to positive level for 10 ms. and then turns it back to zero again.

### To put microcontroller in ultra-low-power mode

Microcontroller is set to ultra-low-power mode and configured to wake-up when RB0 will reach a low value.

# BROADSEIS SRD, PCB drawing